NETWORK INFRASTRUCTURE & SECURITY SPECIFICATION

FOR

Ahd Assem Elgohary(21067349)

Kholoud Ahmed EL-Sokkary (21027261)

Nada Wesam Alhlwany (21029590)

Malak Khaled Hamdy (21092572)

Reem Ayman Abdelaziz (21056379)

Aya Allah Anwar Hamdoun (21083747)

BY

ENTERPRISE NETWORK PROJECT (HQ & BRANCH)

Table of Contents

- INTRODUCTION ........................................................................................................ 3

  - PURPOSE ...........................................................................................................................4

  - SCOPE ................................................................................................................................4

  - Definitions, acronyms, and abbreviations ..........................................................................5

  - Overview ……………………………………………………......................................................................5

- OVERALL DESCRIPTION ............................................................................................. 6

  - NETWORK TOPOLOGY ………………………………………................................................................6

  - VLAN Profile …………...........................................................................................................6

  - Operating Environment .................................................................................................... 7

- SYSTEM FEATURES & FUNCTIONAL REQUIREMENTS................................................. 8

  - Layer 2 Security & Switching ..............................................................................................8

  - IP Addressing & DHCP Management ……………………………………………………………………….8

  - Firewall Policies & UTM Profiles …….…………………………………………………………………………………9

  - SD-WAN & Connectivity …………………………………………………………………………………………………..9

  - Remote Access VPN ………………………………………………………………………………………………………..10

  - IPsec ………………………………………………………………………………………………………………………………10

- WINDOWS SERVER ADMINISTRATION .................................................................... 11

  - Domain Controller Setup …...............................................................................................11

  - User Management …........................................................................................................11

  - Group Policy Objects (GPO) ….......................................................................................... 11

- NON-FUNCTIONAL & QUALITY REQUIREMENTS........................................................ 11

  - TESTING & VALIDATION SCENARIOS...................................................................................12

  - PERFORMANCE REQUIREMENTS………………………………………………………………………………………13

  - Safety Requirements .........................................................................................................13

  - Security Software ………………..............................................................................................14

  - Software Quality Attributes ...............................................................................................15

- TROUBLESHOOTING & CHALLENGES ……………………………………………………………..15

- FINAL TEST & VALIDATION SCENARIOS …………………………………………………………..18

- CONCLUSION……….………………….............................................................................. 19

Part -1

- Introduction

This document outlines the design, implementation, and configuration of a secure enterprise network infrastructure connecting a Headquarters (HQ) and a Branch Office. The project aims to provide a robust, scalable, and secure environment that supports departmental operations through network segmentation, advanced firewall protection, and centralized management.

The infrastructure is built using GNS3 simulation software, integrating Cisco Switches for Layer 2 security and FortiGate Firewalls for Layer 3 routing and threat management . Additionally, a Windows Server 2022 environment is deployed to handle user authentication and policy management via Active Directory.

Key Objectives:

- Connectivity: Ensuring reliable communication between sites using SD-WAN technology.

- Security: Implementing Port Security, MAC Filtering, and Firewall Policies to protect against unauthorized access.

- Management: Centralizing user control and resource access through Active Directory and Group Policies.

- Remote Access: Enabling secure remote work capabilities using SSL VPN.

I.I. Purpose

The purpose of this document is to define the technical requirements and configuration details for a secure enterprise network connecting a Headquarters (HQ) and a Branch Office. The system aims to provide segmentation, high availability, and advanced security against internal and external threats using FortiGate Firewalls and Cisco Switching.

I.II. Scope

The project encompasses the following key deliverables:

- Logical Segmentation: Deployment of VLANs to separate departments (IT, HR, Finance, Sales, Management, Servers) .

- Perimeter Security: Implementation of Firewall Policies with Granular UTM profiles (Web Filtering, Antivirus, Application Control) specific to each department .

- Internal Security: Layer 2 protection using Port Security and MAC Filtering to prevent unauthorized access .

- Connectivity: SD-WAN configuration for intelligent traffic steering and failover between dual WAN links  & Site-to-Site IPsec VPN and pings between different Vlans.

- Remote Management: SSL VPN setup for secure remote access to the management network.

I.III.  DEFINITIONS, ACRONYMS, AND ABBREVIATIONS

| Term | Definition |
| --- | --- |
| SD-WAN | Software-Defined Wide Area Network. |
| SLA | Service Level Agreement (used for measuring link quality). |
| UTM | Unified Threat Management (Antivirus, Web Filter, App Control). |
| Sticky MAC | A port security feature that dynamically learns and saves MAC addresses. |
| Split Tunneling | A VPN feature that routes only specific traffic through the tunnel. |
| VLAN | Virtual Local Area Network. |
| GPO | Group Policy Object (Windows Server). |

I.IV.  Overview

This document details the functional requirements for the network. Section II outlines the Departments. Section III details the specific configurations for Security, Routing, and Switching. Section V provides validation proofs for the implemented features.

- Overall Perspective

II.I. Network Topology

The system operates as a simulated enterprise environment using GNS3. It integrates Cisco IOU Switches for local distribution and FortiGate VMs (HQ & Branch) as the core gateways. The topology includes dual WAN routers (R1, R2) to simulate redundant internet connections.

II.II. Vlan Profiles

The network is designed around specific user groups with distinct access privileges:

- IT Department (VLAN 10):

  - Access: Full Internet Access.

  - Restrictions: Blocked social media (Facebook) and adult content; Allowed tech sites (GitHub, Microsoft) .

- HR Department (VLAN 20):

  - Access: Focus on recruitment platforms.

  - Restrictions: Allowed LinkedIn/Zoom; Blocked Reddit/Entertainment .

- Finance Department (VLAN 30):

  - Access: High-security financial transactions.

  - Restrictions: Strict MAC Address Filtering applied; Allowed Investopedia/Microsoft; Blocked TikTok/Facebook .

- Sales Department (VLAN 40):

  - Access: E-commerce and CRM tools.

  - Restrictions: Allowed Shopify/Salesforce; Blocked YouTube .

- Management (VLAN 50):

  - Access: Executive level, accessible remotely via VPN.

  - Restrictions: High priority Web Filtering .

- Servers (VLAN 60):

  - Access: Restricted inbound/outbound.

  - Security: Heavy IPS and Antivirus profiles .

II.III. Operating Environment

Virtualization

- GNS3 Network Simulator.

Firewall OS

- FortiOS v7.0.9.

Clients

- Windows 10/Server 2022

- Kali Linux (for testing)

- VPCS.

- SYSTEM FEATURES & FUNCTIONAL REQUIREMENTS

III.I. Layer 2 Security

- Port Security:

- Applied on Cisco Access Switches.

- Max MACs: 1 per port.

- Violation Mode: Shutdown.

- Result: Prevents MAC Spoofing and unauthorized device connections .

- VLAN Configuration:

- HQ Subnets: 192.168.x.0/24 (where x = VLAN ID).

- Branch Subnets: 172.16.x.0/24 (where x = VLAN ID) .

III.II. IP Addressing & DHCP Management

- DHCP Server: Configured on FortiGate interfaces for all user VLANs.

- Exclusions: Static IP range reserved for Servers in VLAN 60 (e.g., Server IP 192.168.60.10).

- Relay: IP Helper addresses configured on switches to forward requests to the gateway.

III.III. Firewall Policies & UTM Profiles

- Policy Structure:

- Separate policies created for each VLAN to access the Internet (NAT Enabled) and to access the Internal Server (No NAT).

- A policy was configured on the server/FortiGate to allow user VLANs to communicate with the network servers, with successful ping tests enabled between them.

- MAC Filtering: Specific "Deny" policy created for VLAN 30 (Finance) to block any MAC address not explicitly listed in the "Allowed_MACs" object .

- Security Profiles:

- Antivirus: Flow-based, Block mode (HTTP, SMTP, FTP, CIFS).

- Web Filtering: Allowing / blocking Specific Websites.

- App Control: Categories blocked based on department function (e.g., Gaming blocked for HR).

III.IV. SD-WAN & Connectivity

- Members: Two WAN links (WAN_R1, WAN_R2) configured with Gateways 10.10.10.1 and 10.20.20.1 .

- Performance SLA:

- Targets: 10.10.10.1 and 10.20.2.1.

- Thresholds: Latency 50ms, Jitter 20ms, Packet Loss 5% .

- Rules: "Best Quality" strategy steers traffic to the interface with the lowest latency/loss.

III.V. Remote Access (SSL VPN)

- Portal: Tunnel Mode with Split Tunneling enabled to access VLAN 50 (Management) .

- Authentication: Local user user1 belonging to group SSL_VPN_USERS.

- IP Pool: Dedicated range SSLVPN_TUNNEL_ADDR1.

- Security: Restrictive firewall policy allowing traffic only from ssl.root to VLAN 50.

III.VI. IPsec

The product supports Site-to-Site VPN using IPsec with pre-shared keys (PSK) on FortiGate devices, enabling secure encrypted communication between the HQ and Branch LANs."

Site-to-Site VPN

connects two networks through encrypted IPsec tunnels.

Typical Use Case

HQ LAN (192.168.x.x/24) ↔ Branch LAN (172.16.x.x/24)

Both sites use FortiGate ( IPsec-compatible device)

- WINDOWS SERVER ADMINISTRATION

IV.I. DOMAIN CONTROLLER SETUP

- Server Name: WIN-CH5K1CJCC4R

- Domain: company.local

- IP Configuration: Static IP 192.168.60.10 on VLAN 60. Gateway 192.168.60.254.

IV.II. USER MANAGEMENT

Users correspond to the VLAN structure. For example, "Ahd Elgohary" is in the Management OU, and "Reem Ayman" is in the IT OU.

IV.III GROUP POLICY OBJECTS (GPO)

Specific GPOs (e.g., Sales-Mapped-Drives) were created to map shared network folders automatically for authorized users.

- NON-FUNCTIONAL & QUALITY REQUIREMENTS

V.I TESTING & VALIDATION SCENARIOS

This section describes the verification tests performed to ensure the network infrastructure meets the design specifications.

| Test ID | Scenario Name | Description | Expected Result |
| --- | --- | --- | --- |
| TST-01 | Port Security Violation | Connect an unauthorized device (Kali Linux) to a switch port configured with sticky MAC. | Port state changes to Error-Disable or Shutdown. |
| TST-02 | SD-WAN Failover | Physically disconnect the primary WAN link (WAN_R1) while a ping stream is active. | Traffic automatically reroutes to WAN_R2 with minimal packet loss. |
| TST-03 | Web Filtering Policy | Attempt to access restricted categories (e.g., Social Media) from the Finance VLAN. | Browser displays "FortiGuard Web Page Blocked". |
| TST-04 | Mac Address Filtering | Spoof a valid IP in VLAN 30 but use an unauthorized MAC address. | Traffic is denied by the Firewall Policy. |
| TST-05 | SSL VPN Connectivity | Connect via FortiClient using user1 credentials from an external network. | VPN Tunnel establishes; internal Management resources are accessible. |
| TST-06 | Domain Authentication | User logs in to a domain-joined PC (e.g., company\reem). | Login successful; Mapped Drives appear automatically. |
| TST-07 | Site-to-Site IPSEC VPN | Test IPsec VPN connectivity between HQ LAN and Branch LAN via FortiGate. | Encrypted tunnel is established, traffic passes securely between sites. |
| TST-08 | DHCP from FortiGate | Test DHCP assignment from FortiGate to VLANs/users. | Devices in VLANs receive correct IP addresses from FortiGate DHCP server. |

V.II PERFORMANCE REQUIREMENTS

PER-1: Latency & Jitter: The SD-WAN SLA is configured to ensure critical traffic uses links with Latency < 50ms and Jitter < 20ms .

PER-2: Failover Speed: The network must switch to the backup WAN link immediately upon failure detection (measured in milliseconds) to prevent session drops.

PER-3: DHCP Response: The FortiGate DHCP server must assign IP addresses to new clients within 5 seconds of connection request .

PER-4: VPN Throughput: The SSL VPN tunnel must support concurrent management sessions without significant lag.

V.III SAFETY REQUIREMENTS

  - SAF-1: Redundancy: The network uses dual WAN links (R1 & R2) to ensure no single point of failure exists for internet connectivity .

  - SAF-2: Isolation: Critical departments (Finance, Servers) are isolated in separate VLANs (30, 60) to prevent accidental data leaks or lateral movement of threats.

  - SAF-3: Disaster Recovery: The Windows Server Domain Controller supports database recovery mechanisms (as demonstrated by the DFS replication recovery after dirty shutdown).

V.IV SECURITY SOFTWARE & HARDWARE

- SEC-1: Firewalling: FortiGate Next-Generation Firewall is the primary security appliance, enforcing policies based on IP, User, and Application .

- SEC-2: Threat Management (UTM): Active Security Profiles are applied:

- Antivirus: Flow-based scanning for HTTP/SMTP/FTP.

- Application Control : Categories blocked based on department function.

- Web Filtering: Category-based blocking (Gambling, Adult, Social Media) .

- SEC-3: Layer 2 Protection: Cisco Switch Port Security prevents MAC flooding and spoofing attacks at the edge .

- SEC-4: Authentication: Active Directory (Kerberos) is used for centralized user authentication.

V.V SOFTWRE QUALITY ATTRIBUTES

- SQA-1: Availability: The system ensures 99.9% availability for internet connectivity via SD-WAN load balancing and failover rules .

- SQA-2: Scalability: The IP addressing scheme (Class C subnets /24) and VLAN structure allow for easy addition of new users or departments without reconfiguration.

- SQA-3: Manageability: The network is centrally managed via the FortiGate Dashboard (GUI) and Windows Server Group Policy Management Console (GPMC).

- SQA-4: Integrity: Data integrity is maintained by restricting access to shared folders (Mapped Drives) based on departmental OU membership.

- TROUBLESHOOTING & CHALLENGES

This section documents technical challenges encountered during implementation and their resolutions based on system logs.

| Issue Category | Problem Description | Evidence / Log | Root Cause |
| --- | --- | --- | --- |
| Firewall Licensing | Unable to create new Firewall Policies or Security Profiles. | Error: "Maximum number of entries has been reached". | FortiGate VM Trial License limitation. |
| Routing / RPF | Traffic dropping between VLANs (e.g., VLAN 10 to VLAN 60) despite correct policies. | Debug Log: msg="reverse path check fail, drop". | Reverse Path Forwarding (RPF) check failed. Asymmetric routing occurred in the GNS3 environment. |
| Server Internet | Ping to 8.8.8.8 failed from the Server. | Destination host unreachable. | Missing NAT configuration in the "Servers_to_Internet" policy. |
| Client Connectivity | Ping from Server to Client PCs (e.g., 192.168.10.2) timed out. | Request timed out. | Windows Firewall on client PCs blocking inbound ICMP. |
| DFS Service | DFS Replication service stopped unexpectedly. | Event ID 4012: unexpected shutdown. | VM power loss/Dirty shutdown. |
| SSL VPN Remote Testing | Remote SSL VPN testing from a separate client VM could not be executed. | Browser and PowerShell port tests to 192.168.1.26:10443 succeeded, confirming the service was reachable. | The GNS3 Cloud was mapped to the internal GNS3 virtual network instead of a real NIC, so adding a remote VM required topology changes that risked project stability. |

- Final Test & Validation Scenarios

| Test Case ID | Feature | Description | Expected Result | Status |
| --- | --- | --- | --- | --- |
| TC-01 | Port Security | Connect unauthorized Kali Linux to switch port. | Port Status changes to Secure-shutdown. | PASSED |
| TC-02 | SD-WAN Failover | Disconnect WAN_R1 link during continuous ping. | Traffic switches to WAN_R2 with <1s downtime. | PASSED |
| TC-03 | MAC Filtering | Spoof IP in Finance VLAN with unknown MAC. | Traffic blocked by Firewall Policy. | PASSED |
| TC-04 | SSL VPN | Connect via FortiClient as user1. | Tunnel establishes; Ping to VLAN 50 successful. | Partial / Not Executed (due to limitations) |
| TC-05 | Server Access | Ping Server 192.168.60.10 from User VLANs. | Ping Reply received (Internal routing functional). | PASSED |
| TC-06 | IPsec VPN | Ping any PC in any vlan in HQ to BR | ping reply received (tunnel is up) | PASSED |
| TC-07 | DHCP | Write " ip dhcp" at Console of pc | then will be written DDORA and given IP dhcp from fortigate and GW | PASSED |

- CONCLUSION

The Enterprise Secure Network Project has successfully achieved its primary objective: designing and deploying a robust, scalable, and highly secure infrastructure connecting Headquarters (HQ) and Branch offices. By integrating Cisco switching, Fortinet security, and Microsoft server administration, the project demonstrates a complete "Defense-in-Depth" strategy.

Key Achievements & Deliverables:

- Network Segmentation & Traffic Control:

  - The network was logically divided into six distinct VLANs (IT, HR, Finance, Sales, Management, Servers), ensuring granular control over broadcast domains and departmental access .

  - Inter-VLAN routing and internet access were strictly regulated via FortiGate Firewall Policies, ensuring that sensitive departments like Finance and Servers are isolated from unauthorized traffic .

- Multi-Layered Security Architecture:

  - Layer 2 Protection: The implementation of Port Security on Cisco switches successfully mitigated local threats such as MAC Spoofing, automatically disabling ports upon violation .

  - Layer 3 Identity: MAC Address Filtering on the FortiGate firewall provided an additional authentication layer for critical zones, ensuring only authorized hardware could communicate .

  - Layer 7 UTM: Comprehensive security profiles (Web Filtering, Antivirus, Application Control, and IPS) were applied to enforce corporate usage policies and block external threats .

- High Availability & Business Continuity:

  - The SD-WAN deployment with "Best Quality" rules provided intelligent traffic steering across dual WAN links. Failover tests confirmed zero-downtime switching between providers during link degradation, ensuring continuous business operations .

- Centralized Management & Administration:

  - The deployment of Windows Server 2022 as a Domain Controller (WIN-CH5K1CJCC4R) centralized user authentication and resource management.

  - Group Policy Objects (GPOs) automated the mapping of network drives, streamlining the user experience based on departmental membership (OU structure).

- Secure Remote Access:

  - SSL VPN (Tunnel Mode) was successfully configured, allowing authenticated staff to securely access the Management network from external locations while maintaining strict access controls .

Resolution of Technical Challenges: The project team successfully diagnosed and resolved critical implementation hurdles, including Asymmetric Routing (RPF failures) in the dual-link environment, VMware/DFS replication errors, and Firewall licensing constraints. These troubleshooting efforts ensured a stable and functional simulation environment.

Final Verdict: The implemented infrastructure meets modern enterprise standards for Confidentiality, Integrity, and Availability (CIA Triad). It stands as a verified, secure, and manageable solution ready for production deployment.