Project Documentation – Network Topology & VPN Setup

# Introduction

This project demonstrates how to connect two branch/company networks securely over the internet using FortiGate firewalls. Each site has multiple WAN connections (TE-Data, Vodafone, and Leased-Line), and connectivity is optimized through IPsec VPN and later SD-WAN for performance, load balancing, and redundancy.

# Objectives

- Configure FortiGate firewalls to provide internal access.

- Implement DHCP to assign IP addresses dynamically to hosts.

- Establish a secure IPsec VPN tunnel between both branches.

- Implement SD-WAN to ensure reliability and improve WAN utilization.

# Network Topology Overview

- FortiGate Firewalls (FG-1 & FG-2): Provide security and connect each branch to multiple ISPs.- Internet Switch: Aggregates connections from ISPs (TE-Data, Vodafone, Leased-Line) to the FortiGate.

- LAN Switches: Switch1 & Switch2 at Branch 1, Switch3 & Switch4 at Branch 2.

- End Devices: Servers (Server-1 in Branch 1, Server-2 in Branch 2), PCs across VLAN10/20/30, IP Phones connected at each switch for VoIP services.

# VLANs

- VLAN 10: Users-1 (Branch 1), Users-4 (Branch 2)

- VLAN 20: Users-2 (Branch 1), Users-5 (Branch 2)

- VLAN 30: Users-3 (Branch 1), Users-6 (Branch 2)

# Network Design Summary

- Each branch is secured by a FortiGate firewall, providing internal and external connectivity.

- Internally, users, servers, and IP phones are distributed across VLANs and LAN switches.

- Externally, internet access is available through multiple ISPs.

- A secure IPsec VPN tunnel connects both branches for inter-company communication.- SD-WAN will later be configured for smart WAN usage (failover + load balancing).

# Implementation Steps

1. Interface Configuration

- Assign IP addresses to WAN & LAN interfaces on FortiGate.

- Connect WAN ports to internet switches and LAN ports to internal switches.

2. DHCP Setup

- Enable DHCP on LAN interfaces

- Define IP ranges for each VLAN.

3. Static Routing

- Add static routes if required to direct traffic through correct WANs.

4. IPsec VPN Setup

- Phase 1: Configure authentication (Pre-Shared Key), public IPs, and encryption settings.

- Phase 2: Define internal subnets to be exchanged securely.

5. Firewall Policies

- Allow VPN-to-LAN and LAN-to-VPN traffic.

- Ensure policies allow communication between internal VLANs and VPN tunnels.

6. Testing

- Verify connectivity with ping tests between Branch 1 and Branch 2 devices.

# Conclusion

This documentation outlines the design and implementation of a secure IPsec VPN connecting two branches using FortiGate firewalls. The setup provides dynamic IP management (DHCP), secure communication across VLANs, and reliable internet access via multiple ISPs. Future enhancement with SD-WAN ensures optimal WAN performance, redundancy, and high availability.